The MIC-1 bit format is shown below. You should be familiar with all the fields and how they are used. Also below are 4 MAL instructions. Indicate if a given MAL is valid or invalid for MIC-1, and if valid, fill in the DECINAL (i.e. bits 1101 are filled as 13) values for each field in the space provided. If invalid, write below the figure why not, but in case you are wrong fill in as many of the fields as you can.

Register designations are as follow:

pc = 0 (program counter)        ac = 1 (accumulator)                                 sp = 2 (stack pointer)

ir = 3 (instruction register)      tir = 4 (temporary instruction register)     zr = 0 (fixed zero)

po = 6 (plus 1)                         no = 7 (minus 1)                                       amask = 8 (address mask)

smask = 9 (stack mask)           a = 10 (a scratch)                                      b = 11 (b scratch)

c = 12 (c scratch)                     d = 13 (d scratch)                                     e = 14 (e scratch)

f = 15 (f scratch)

- pc := pc + 255; mar := pc; rd

- ac := inv(mbr); mar := inv(mbr); wr;

- tir := lshift(band(tir, mbr)); if n then goto 150

- mbr := lshift(band(ir, amask)); ir := lshift(band(ir, amask)); goto 0; wr;

- b := ir – ac; mar := ac; if z then goto 158;

| MAL | Valid? | AMUX | COND | ALU | SH | MBR | MAR | RD | WR | ENC | C | B | A | ADDR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | yes | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 9 | - |
| B | no | - | - | - | - | - | - | - | - | - | - | - | - | - |
| C | yes | 1 | 1 | 1 | 2 | 0 | 0 | 0 | 0 | 1 | 4 | 4 | - | 150 |
| D | yes | 0 | 3 | 1 | 2 | 1 | 0 | 0 | 1 | 1 | 3 | 8 | 3 | 0 |
| E | no | - | - | - | - | - | - | - | - | - | - | - | - | - |

AMUX	COND			ALU			SH

0 = latch	0 = no jump		0 = A + B		0 = no shift

1 = MBR	1 = jump if n = 1	1 = A AND B		1 = shift right

2 = jump if z = 1	2 = A			2 = shift left

3 = always jump	3 = not A

MBR, MAR, RD, WR, ENC

0 = no

1 = yes